Project Documentation: CRICKET WORLD CUP 2027

Group 1506

1. Assumptions Made

- Base Network Calculation: According to the project rules, the last 4 digits of a group member's ID are used to form the base network. Using ID , the last 4 digits are 1004. This makes our base network 10.4.0.0/16.

- Topology Interpretation: The instructions state Pakistan, Sri Lanka, and Bangladesh are connected via a shared switch. This creates a transit LAN. The instructions also state a "loop" exists (BD-IN, IN-SL, SL-PAK, PAK-BD). Since SL, PAK, and BD are already connected via the shared switch, we used direct Point-to-Point serial links for BD-IN and IN-SL, while the shared switch fulfills the loop requirement for the rest.

- Central DNS: To ensure all devices can access www.india.cricket.com and www.bangladesh.cricket.com, a central DNS Server was placed in the Pakistan LAN. All DHCP scopes across the topology point to this server to resolve hostnames.

- Recursive Routing Next-Hop: For India's recursive backup route to Bangladesh, India points to Bangladesh's Shared Switch interface IP. If the direct BD-IN link fails, India recursively looks up this IP and routes the traffic through Sri Lanka.

2. VLSM Tree / Subnetting Calculation

Networks were ordered from largest to smallest host requirements to minimize IP waste.

Plaintext

10.4.0.0/16 (Base Network)

├── 10.4.0.0/23   [India LAN | 510 hosts]

├── 10.4.2.0/24   [Pakistan LAN | 254 hosts]

├── 10.4.3.0/24   [Bangladesh LAN | 254 hosts]

├── 10.4.4.0/24   [Sri Lanka LAN | 254 hosts]

├── 10.4.5.0/24   [Further Subdivided Below ↓]

│    ├── 10.4.5.0/25       [Afghanistan LAN | 126 hosts]

│    └── 10.4.5.128/25     [Further Subdivided Below ↓]

│         ├── 10.4.5.128/29   [Shared Switch (PAK,SL,BD) | 6 hosts]

│         ├── 10.4.5.136/30   [Link: BD - IN | 2 hosts]

│         ├── 10.4.5.140/30   [Link: IN - SL | 2 hosts]

│         ├── 10.4.5.144/30   [Link: PAK - AFG | 2 hosts]

│         └── 10.4.5.148/30 - 10.4.5.255 [UNUSED IP BLOCKS]

└── 10.4.6.0/16 through 10.4.255.0/16 [UNUSED ENTIRE SUBNETS]

| Hotel / Link | Required Hosts | Subnet Mask | Allocated Network | Broadcast Address | Usable IP Range |
| --- | --- | --- | --- | --- | --- |
| India LAN | 300 | /23 | 10.4.0.0 | 10.4.1.255 | 10.4.0.1 - 10.4.1.254 |
| Pakistan LAN | 220 | /24 | 10.4.2.0 | 10.4.2.255 | 10.4.2.1 - 10.4.2.254 |
| Bangladesh LAN | 180 | /24 | 10.4.3.0 | 10.4.3.255 | 10.4.3.1 - 10.4.3.254 |
| Sri Lanka LAN | 150 | /24 | 10.4.4.0 | 10.4.4.255 | 10.4.4.1 - 10.4.4.254 |
| Afghanistan LAN | 120 | /25 | 10.4.5.0 | 10.4.5.127 | 10.4.5.1 - 10.4.5.126 |
| Shared Switch (PAK, SL, BD) | 3 | /29 | 10.4.5.128 | 10.4.5.135 | 10.4.5.129 - 10.4.5.134 |
| Link: BD - IN | 2 | /30 | 10.4.5.136 | 10.4.5.139 | 10.4.5.137 - 10.4.5.138 |
| Link: IN - SL | 2 | /30 | 10.4.5.140 | 10.4.5.143 | 10.4.5.141 - 10.4.5.142 |
| Link: PAK - AFG | 2 | /30 | 10.4.5.144 | 10.4.5.147 | 10.4.5.145 - 10.4.5.146 |

3. Comprehensive IP Addressing Table

All end-devices (PCs) receive IPs dynamically via Router-Based DHCP and are not listed as static. Local and Central Servers use Static IP assignments.

India Hotel (DHCP IP Assignment for PCs)

- Router Gateway (LAN): 10.4.0.1

- Web Server (Static): 10.4.0.2

- Email Server (Static): 10.4.0.3

- WAN to Bangladesh: 10.4.5.138

- WAN to Sri Lanka: 10.4.5.141

Bangladesh Hotel (DHCP IP Assignment for PCs)

- Router Gateway (LAN): 10.4.3.1

- Local Web/Email Servers (Static): 10.4.3.x

- WAN to India: 10.4.5.137

- WAN to Shared Switch: 10.4.5.129

Sri Lanka Hotel (DHCP IP Assignment for PCs)

- Router Gateway (LAN): 10.4.4.1

- Local Servers (Static): 10.4.4.x

- WAN to India: 10.4.5.142

- WAN to Shared Switch: 10.4.5.130

Pakistan Hotel (DHCP IP Assignment for PCs)

- Router Gateway (LAN): 10.4.2.1

- Central DNS Server (Static): 10.4.2.5

- Local Servers (Static): 10.4.2.x

- WAN to Afghanistan: 10.4.5.145

- WAN to Shared Switch: 10.4.5.131

Afghanistan Hotel (DHCP IP Assignment for PCs)

- Router Gateway (LAN): 10.4.5.1

- Local Servers (Static): 10.4.5.x

- WAN to Pakistan: 10.4.5.146

4. Configuration Commands

Afghanistan Router (Default Route Only):

enable

configure terminal

! Default route pointing all traffic to Pakistan

ip route 0.0.0.0 0.0.0.0 10.4.5.145

do write

Pakistan Router (Static Routing & Local DHCP):

enable

configure terminal

! Protect the gateway and central DNS server IPs

ip dhcp excluded-address 10.4.2.1 10.4.2.10

! Local DHCP Configuration for Pakistan (Points to local DNS)

ip dhcp pool PAK_LAN

network 10.4.2.0 255.255.255.0

default-router 10.4.2.1

dns-server 10.4.2.5

exit

! Route to Afghanistan

ip route 10.4.5.0 255.255.255.128 10.4.5.146

! Routes to Asian Loop (via Shared Switch)

ip route 10.4.3.0 255.255.255.0 10.4.5.129

ip route 10.4.0.0 255.255.254.0 10.4.5.129

ip route 10.4.4.0 255.255.255.0 10.4.5.130

do write

Bangladesh Router (RIPv2, Redistribution & Central DHCP for BD/SL): Acts as central intelligence by redistributing static routes into RIP.

enable

configure terminal

! Central DHCP for Bangladesh (Points to Pakistan DNS)

ip dhcp excluded-address 10.4.3.1 10.4.3.10

ip dhcp pool BD_LAN

network 10.4.3.0 255.255.255.0

default-router 10.4.3.1

dns-server 10.4.2.5

exit

! Static routes to networks RIP doesn't touch

ip route 10.4.2.0 255.255.255.0 10.4.5.131

ip route 10.4.5.0 255.255.255.128 10.4.5.131

! RIP Configuration

router rip

version 2

no auto-summary

network 10.4.3.0

network 10.4.5.128

network 10.4.5.136

redistribute static

exit

do write

Sri Lanka Router (RIPv2 Only):

enable

configure terminal

router rip

version 2

no auto-summary

network 10.4.4.0

network 10.4.5.128

network 10.4.5.140

exit

do write

India Router (RIPv2 + Recursive Backup Route):

enable

configure terminal

router rip

version 2

no auto-summary

network 10.0.0.0

exit

! Floating Recursive Route to Bangladesh if BD-IN link fails.

! Points to BD's Switch IP. Since IN knows the switch IP via SL, it routes via SL.

ip route 10.4.3.0 255.255.255.0 10.4.5.129 130

do write